GIRISH HARNESSES LTD (GHL)

# Company history and background

Girish, the present owner of GHL graduated as a mechanical engineer in 1992. He worked for a year in industry and after that started his own trading business in Aurangabad. In January 1996, based on a friend’s advice, Girish approached Indian Harnesses Ltd.(IHL)  – a leading harness assembler and supplier for the Indian automobile industry. Unfortunately, IHL did not show much interest. IHL used to supply wiring harnesses to large 4-wheeler and 2-wheeler manufacturers in India, like Japani Cars Ltd, Indian Scooters Ltd., and also for export. IHL assembled harnesses and also manufactured its key components like connectors and terminals.

The year 1996-97 saw financial trouble for IHL as their export orders declined due to recession in South-East Asia. As a strategic initiative IHL decided to focus on assembly of harnesses only for the 4-wheeler segment, and to off-load its 2-wheeler business to a third party for which it invited Girish for negotiation. The two sides spent the year 1997-98 in working out mutually agreeable details of the deal. A joint venture called GHL was formed by mid 1998.

By November 1998 GHL’s plant was built and the machinery was installed. IHL helped GHL to buy machines. GHL began by supplying harnesses for the ‘K-4S’ motorcycle model of Indian Scooters Ltd. Later, as the sales of the new ‘Cal’ model of Indian Scooters increased, it encouraged GHL to begin the supply of harnesses for that model. In April 1999 GHL also got orders from another scooter manufacturer - Kinetic Scooters Ltd. Volumes of orders from Indian Scooters and Kinetic Scooters increased rapidly from 100-200 harnesses per day in Jan-Feb 1999 to 500 harnesses per day in April 1999. These orders are expected to increase further to 1000-2000 harnesses a day by June-July 1999.

Based on existing capacity of machines and the employed labour, GHL is barely able to assemble 500 harnesses a day, operating in a single shift of 8 hours that extends usually to 10-11 hours for certain work areas. Girish is worried about the rapidly increasing orders. He has tried a few methods to enhance the productivity of his existing plant, but the productivity has not improved significantly. Currently many of his operations are manual. There is no formalised production planning, though manual records of production and quality related data are maintained. Girish feels that there is a lot of scope for implementation of  formalised operational systems aided by information technology once GHL’s orders and production have stabilised. So far he has been taking decisions in reaction to immediate pressures. However the magnitude and complexity of these pressures are increasing fast, and Girish wants a formal plan for i) improving the effectiveness and productivity of his existing operations and practices over short to medium term, ii) expanding GHL’s capacity of machines, labour and space over medium to long term, and iii) setting formal systems in long term that will enable him to grapple with changing demands from the customers.

Investment and finances

The venture has been set up as a joint venture, with 26 percent equity from IHL, 74 per cent from Girish (about Rs 20 lakhs), and a loan from a co-operative bank (Rs 40 lakhs) and some money for working capital. Of all this, the setting costs themselves amount to Rs 20 lakhs for land, building and land development, Rs 40 lakhs for plant and machinery and other pre-operating expenses of Rs 4 lakhs.

There are two other small scale sector benefits that GHL has availed of. One is a loan from the state government to support the entrepreneur¹s equity stake, and the other is a sales tax exemption for all products manufactured in a certain time period (10 years), provided this is used for financing the growth of the company.  These are significant incentives.

The plant has been set up with the premise that firm orders from a major customer are guaranteed.  This is the case as of now, and in fact, the major customer, sensing a cost advantage, and a keen, fresh entrant into the business, is pushing GHL to expand.  The financing of such an expansion at an early stage is one of the concern areas for GHL.  Even though the relationship with the major customer is positive, certain credit arrangements continue to hold (e.g. 90 days for payment).  On the other hand, being a relatively small player, GHL does not have as much leverage with some of its key suppliers (e.g. terminal and connector suppliers).  Wages have to be paid, and some new equipment purchase has to be made to ensure sufficient capacity, and increasingly, reliability of supply.

# The Product – Wiring Harness for automobiles

Wiring harnesses in automobiles are used for transmitting electrical energy to various electrical fixtures like front lamps, rear lamps, inside lamps, air-conditioner, radio/audio cassette player. A wiring harness is a sub-assembly that consists of PVC cables, terminals (metal strips attached at the ends of a cable), connectors (plastic end fitments for terminals), PVC tube (to cover the cables), adhesive tape (to bunch together cables), fuse box and transparent sleeve. Although simple by design, it is critical sub-assembly for the automobile. Important quality parameters are the continuity of cables and the tightness of clamping of terminals with connectors.

K-4S harness has 48 cables and 65 circuits, while the Cal harness has 52 cables and 75 circuits. Large number of cables and circuits makes the control of quality a complicated job, particularly since a large part of the assembly process is manual. GHL buys the critical components (connectors and terminals) from IHL, and the remaining components from local suppliers. Cal harness requires waterproof connectors that GHL imports from Korea. In future IHL plans to manufacture them in-house and sell to GHL.

# Organizational Structure at GHL

Girish has two more senior managers looking after the business development and overall management. All three partners deal with the customers, suppliers, and also closely monitor the in-house operations. Then there is a plant engineer who is always present in the plant and closely monitors and controls the production/assembly processes. The engineer is responsible for the quality and maintains manual records of daily production and quality defects. The engineer also works on design of jigs and fixtures for the new models (like Cine Fonda). About 25-30 workers work in the manufacturing/assembly. Most of the workers are temporary, and trained in-house by the engineer.

# Existing operations and practices at GHL

## Manufacturing processes and capacities

Exhibit I shows the manufacturing process sequence and layout at GHL. The process sequence can be thought of as comprising creation of cables (consisting of cutting, stripping, crimping, and taping of joints), assembly operations (comprising fitting of connectors and taping of cables), and testing/packaging This process was mostly borrowed from the process employed at IHL. However there are some differences from the process at IHL since IHL assembles harnesses for 4-wheeler segment where volumes are lower and complexity is higher compared to harnesses for 2-wheelers. Unlike IHL, the sub-assembly process at GHL involved fitting of connectors for all cables in a harness at the same work-table by the same operator. At IHL this job is divided into certain elements and performed in a sequence of work-tables by different operations. This reduces the complexity of job at every work-table. At GHL every workers in sub-assembly area had to insert more than 50 cables in 10-15 connectors, and bunch cables with tapes.

The existing capacities of the operations at GHL are shown in Exhibit II. The raw material for GHL, including cables of length 1 m or greater, connectors, terminals, tapes etc are stored in an area close to the shop-floor. Typically one month’s inventory of raw material is maintained in order to hedge against uncertainty of supply. The first operation is cutting and stripping which is done on a single automatic machine. A worker has to feed in the cables and operate a switch. This machine can cut 12000 to 16000 cables in a shift depending of the initial cable length. This is equivalent to 240 to 320 harnesses a shift. Next operation is crimping for which there are 5 semi-automatic machines. Crimping is attaching a terminal to the end of a cable by pressing. Each of these machines can perform 18000 to 22000 (????) crimps per shift. Each machine requires a worker who picks a cable (or two cables in case of joint crimping for two cables), holds the cable(s) carefully under the tip of pressing tool, and then switches on the machine to perform the pressing operation. The capacity of crimping operation at each of the 5 machines is equivalent to about ??????.

Next major operations were sub-assembly followed by final assembly. Both were manual and performed on work-tables where a worker on a table had to pick each cable and insert it in appropriately coloured connector (in case of sub-assembly) or place the harness with proper shape over the fixture and tape it accordingly (in case of final assembly). The sub-assembly operation was intricate and required concentration on part of the worker as he/she had to pick about 50 cables and insert them in about 10 to 15 connectors by matching the colours. Each work-table at sub-assembly and final assembly had a capacity of about 100 harnesses per shift, though it varied significantly depending on the skill of the worker. An inexperienced worker could make only 50-60 harnesses on a sub-assembly table, while an experienced and skilled worker could make more than 100 harnesses in a shift.

Finally, quality inspection was performed in two stages – visual inspection and continuity inspection on different tables. Each of these inspections could be performed on about 250 harnesses by a single worker. There were 3 tables each for visual and continuity inspections, though at present two workers were engaged in each kind of inspection.

## Quality systems

Following the recommendation of the major customer, GHL follows the proper display of the process sequence and maintains PCQC (process control quality chart) reports.  The various processes are all listed, and inspection parameters and responsibility has been identified on paper.  The plant engineer is mainly responsible for maintaining quality records, diagnosis and taking corrective actions.

Defective finished products detected were about 25% in March and 10% in April 1999.  Typical defects were short cable length, wrong or loose connection of terminals with connectors, improper taping, and improper crimping.

## Employee practices

GHL has sought the services of two trained supervisors from IHL.  They are primarily responsible for the setting up of the work system at the shop floor level, and are available for training new workers, any trouble shooting and maintaining production norms and quality targets.

The approximately 30 strong work force (about 5 women, 25 men) is a young one, and for almost all the workers this is their first job. The workers are literate and have education up to about 10th standard, and live in three nearby villages.  The skill level of workers varies significantly because of the difference in experience and high manual content of certain operations.  Girish uses ‘piece-rate’ incentive scheme to induce workers to increase their speed of work.  Targets are set for every worker in the sub-assembly area, based on his current performance, every day.

## Supplier policy

GHL has the following major bought out items:

1) Cables: These are wires enclosed in plastic which are colour coded. There are two suppliers, one in Hyderabad and one in Daman

2) Terminals: These are the brass contact points, which are attached to cable ends during the crimping operation.  These are supplied by IHL.

3) Connectors: These are moulded plastic enclosures for the terminals, which are presently supplied by a foreign supplier with a base in India. This supply is so far reliable, but GHL¹s partner IHL is planning to take up this manufacture itself.  This is a component where there are some product development issues as well, such as developing  water-proof connectors.

4) Other components, such as fuses, rubber/PVC sleeve, tapes, seals and gaskets.

The supplies so far are reliable, and in its initial phase, GHL is using a highly conservative policy of having raw material stocks of up to a month, to avoid any disruption in supply.

Exhibit I : Manufacturing Process sequence and Layout at GHL

Store

Cut &

strip

C1

C2

C3

C4

C5

Subassembly stations

1

2

3

4

5

# Assembly + Tape Stations

1		2							3		4							5		6							7		8										9		10				Visual inspection

Final testing

Packaging and BSR

OFFICE

R & D

Exhibit II : Existing Process Capacities at GHL

| Process | Automatic/ semi-automatic/ manual | # Capacity per shift* | No. of machines/ tables | No. of operators |
| --- | --- | --- | --- | --- |
| Cutting and Stripping | Automatic | 12000 to 16000+ cables/machine | 1 | 1 |
| Crimping | Semi-automatic | 18000 to 22000++ (????)crimps/machine | 5 | 5 |
| Sealing of joints | manual | - | 1 | 1 |
| Sub-assembly (fit connectors and bunch with tapes) | manual | 100 harnesses/table | 10 (5 in use) | 7-8 |
| Final assembly (give shape to harness on the fixture, and tape the cables) | manual | 100 harnesses/table | 24 (6-8 in use) | 6-8 |
| Visual inspection | Manual | 250 harnesses/table | 3 (2 in use) | 2 |
| Continuity inspection | Semi-automatic | 250 harnesses/table | 3 (2 in use) | 2 |
| * One shift was 8 hours. However, workers at certain stations worked for up to 10-12 hours. + Rate of cutting & stripping machine depends on the length of cable. It can cut & strip 16000 cables of length 1 m each or 12000 cables of length greater than 1 m each. ++ Rate of crimping process varied depending on the type of crimping to be done – at the ends or at joints. |  |  |  |  |

CASE ANALYSIS

The firm GHL has growth plans that depend crucially on its managerial and work force, and capital resources.  This is because of the following reasons: The process technology does not permit too much automation, and the work content is very labour intensive, although not necessarily highly skilled.  The location of the plant, and one of the cost advantages of the firm is that a relatively inexpensive work force base is available locally, in nearby villages.  This workforce has to be appropriately trained and used in the manufacturing process.  Further, the managerial resources are already strained and will be further stretched in the months to come.  Now, the managerial presence on the shop floor is strong, variety of orders is low and volumes are manageable, and the absence of hierarchies makes it easy to respond to problems.  This may not continue for long as the variety and volume of orders increase.  Although there would be some scope for improvement in the existing system, the rapidly growing demand implies that very soon Girish would have to contemplate capacity expansion in terms of buying more machines, hiring more labour, buying additional space and ordering further construction of facilities.

There is therefore the need for a strong and proactive policy in all these areas.  Following sections discuss the improvements in existing systems and practices, followed by longer-term recommendations for capacity expansion and instituting formalised planning systems.

Improvement in systems and practices

Work methods:

The work in sub-assembly area is low-skilled but highly manual and involves concentration while matching and inserting terminals into connectors.  Making a single worker do this entire job for about 50 cables at a time (as is the existing practice) could lead to lower inefficiency (and so lower rate of work) and higher incidents of quality problems.  The existing practice would also result in wide disparity in the performance of different workers in the sub-assembly area.  This is because slight difference in skill and experience of two workers would result in magnified difference in daily outputs of the two workers since they are working independently in a highly manual detailed operation.  This is proven by the fact that existing rates of workers in this area vary between 60 and more than 100 harnesses per day.  This also leads to the problem of setting standard incentive norms.  Setting piece rate incentive scheme will add to the disparity by rewarding few workers and de-motivating many.  Such work practice would not be healthy when orders increase and the entire group of workers would be expected to perform well.  Breaking the existing work at a work-table in sub-assembly area into 3-4 sequence of work elements would result in following effects:

- It will reduce the disparity in the overall rates of different workers.  Overall group rate can be improved by training the slow workers.

- Incentives could be based on productivity of the line.  This would encourage enhanced group efficiency.  This would also encourage more productive workers to train their less productive colleagues by sharing their innovative methods.  On the whole, the productivity of the area would increase.  This would be important particularly when the pressure of increasing orders rises.

- The proposed method would have two risks – initially the more productive workers might get frustrated because their productivity would be determined by that of a slower worker, and there could be inventory pile-ups between the work-tables.  Both these risks can be minimised by devising proper a incentive scheme that gives credit to group productivity (not individual productivity) and ensures that the most productive worker gets at least as much as he/she was getting in the earlier system.

- The new system may also call for hiring more workers in the sub-assembly area.  This should not be a problem as the overall productivity would increase and the demand is anyway  increasing at a rapid rate.

The employee practices touched upon here will also be discussed later in the section ‘Employee practices’.

Production planning, Quality & control:

At present the production planning and coordination seems to be easy as there are few products (just two), volumes are manageable, and various processes are almost balanced.  The problems will arise when product variety and volumes increase, as they are bound to.  When more than one product is being done in the shop, the best answer is to have separate lines for each product, wherever possible. This is being done for the proposed scooter line.  But this will not be possible for each and every product variety.  In that case it would make sense to group different products under few families that are vastly ‘different from each other’ but ‘similar within’ on parameters like product design, complexity, type of operations required, type of raw materials required, and criticality in terms of quality expectations of the customer (e.g., export orders, if they come in future and are in significant volumes, need not be mixed with the domestic orders).  When two products are planned on the same line, a clear policy needs to be spelt out to avoid confusion, and also to reduce the batching inventory.

With increased volumes the working capital requirements would also go up and the company would need to guard against holding too much unnecessary stock, especially in raw materials. The present levels of raw material inventory are high (about one month).  This will create problems of costs and become unmanageable when the orders increase in variety and volumes.  Perhaps a simple forecasting system along with a correlation of advance orders with procurement lead times could lead to some savings.  This could be part of a formal planning system, discussed later.  Another reason for high raw material stocks is probably a high amount of uncertainty of reliable supply.  A simple analysis of supply reliability in the last few months, plus some negotiation with suppliers should give enough confidence to carry out a reduction in the raw material inventory holding.

Although a process quality measurement system (PCQC) is in place at present, its full potential is not being realised.  Presently, quality records of different work/inspection areas are maintained by the supervisor, and action taken if performance on certain quality parameter deteriorates.  This system can be strengthened by making the workers directly responsible for the quality of their output.  This has to be supplemented by giving the workers greater control / free-hand over managing the quality of their output by training them appropriately about managing various quality aspects of the product and the machines (process).  Of course, the incentive scheme should  accordingly be designed so as to encourage all the workers in a works area to cumulatively produce high output of good quality.  In areas involving manual work, like the sub-assembly area, a few senior workers who are most productive could be asked to assure the quality of their line in addition to doing the regular work of assembly.  The job roles would have to be re-defined for this.

Managing the inventory of raw material as well as work-in-process (WIP) would require a systematic and formal approach once orders increase in variety and size.  Having formal norms for in-coming stock levels, purchase order replenishment levels, WIP limits between work stations, production order release levels (for major work areas) would help in cutting down on inventory, smoothening the work flow, as well as reducing lead times / cycle times for meeting the orders.

Need for a formal operational planning system:

Does the firm need a formal system at this time?  Given the growth plans of the company, the answer seems to be yes.  It can start off as a simple material control system, to help in raw material procurement, planning and control of material receipts, and payments.  This procurement should not be done in an ad-hoc manner, but as far as possible, in tune with specific or forecasted requirements of various end items.  If the product structure is studied, common items should be netted and inventory reductions (at least of the safety stock variety) should be brought down substantially.  This should be possible as many items like cables, terminals, connectors, tapes, etc would be similar for different products (products differ mainly in the circuitry routings and connections).  The material control system could then be supported by developing a simple but formal demand forecasting system.  Such a demand forecasting would be based on analysis of past data of consumption of different group of items.

Employee practices:

Employee practices are discussed under following heads:

1) Worker wages, production norms and incentives:	Already, the production capacity of the unit is being tested to the limit of single shift operations, with the installed machinery and assembly stations.  Work practice standardisation is ongoing, although there is no formal time study.  In order to achieve higher norms, the management is considering various options, one of which is incentives based on piece rates.  In a multi-stage operation with widely differing cycle times (there is one cutting machine, 5 or so crimping machines, 5 subassembly stations, 4 assembly stations, 2 manual testing stations and one final testing station). Individual piece rates will almost certainly cause flow imbalances with all its problems of excess inventory and shop congestion.

On the other hand, increasing group targets beyond a point, for a fixed salary also may be unreasonable.  Given that the plant would want the workers to learn quickly and the produce good quality output at a fast rate, some quantity linked incentive may be inevitable.  Some group productivity norms could be tried, and some time be allotted for training and planning of activities within each group.

2) Worker flexibility, job rotation and multi-tasking:	As a matter of policy, the work force should be trained for cross functional flexibility.  Right now, probably the management itself is so trained, and that should continue to the extent possible.  The benefits of cross training and planned job rotation are many: potential flexibility in case of absenteeism or any other reason, better understanding of the overall product characteristics, some reduction of job fatigue through repetitive action.

Multi-tasking can be defined with respect to the current set up by noting that a sub-assembly worker and an assembly worker, in particular, are responsible for several operations on a single product.  This has the advantage that material handling is reduced, and that single point (or restiricted to one or two wokers) quality responsibility is possible for a product subassembly.  But in terms of job complexity and possibility of errors, one can consider the breaking up of tasks, especially in the subassembly area.

3) Gain sharing, unionisation, and worker benefits:	GHL is now one of the few firms in this greenfield site, and has provided good, clean conditions for managers and workers to work in.  This should be continued.  At some point, some form of worker organisation is to be expected, especially with the growth plans of the firm.  The firm should not necessarily view this in a negative light, as splitting up the unit, or artificially preventing growth is likely to be managerially even more challenging than the present set up.	As long as the workforce feels that the management is committed to grow and support them, they should have no reason to disrupt the system.  The economic conditions are such that jobs in a firm with growth potential

are hard to come by, anyway.

4) Training:	It is clear that training and familiarity with the job makes a big difference in production outputs.  With new workers needed for the new lines and allowing for some turnover of the workforce, investment in job training will become crucial.  If some group norms are in force, it makes sense to allow senior or long standing workers to train newcomers and less skilled workers.  This can be part of the creative activity which can be suitably valued.

5) Supervisory structure:	For now, two experts from the joint venture partner are performing the job of training and job description.  Over some period of time, a formal structure may need to emerge, to oversee not only the process and work allocation, but several related activitites like planning, material issue and quality monitoring.  It is best if such skills are available with all workers who can then even rotate the job of supervision.  The advantage of this is that the hierarchy is less and leads to responsibility of all concerned.  This naturally means hiring literate and skilled workers, or making them so over a period of time. Broadly speaking, the management would be trying to provide challenges and opportunities for its senior people so that (a) their value - both in earnings and potential - increases over time and (b) their commitment to the organisation continues, so that their contribution is in keeping with company objectives.  It is not clear why this argument cannot be extended to all levels of the workforce.  One can devise a more control oriented mechanism, but this will involve its own costs and inefficiencies.

Information systems for operations planning & control:

Among the many subjects Girish must tackle, perhaps information system for his operations is one of the daunting issues.  Information systems can alter a company's efficiency and productivity. Though Girish is convinced of I.T./I.S.’ ability to cut costs & improve planning, computerised systems are still uninitiated as they are seen as a complex array of options that are difficult to understand and master. In the next few paragraphs we discuss the various issues in I.S. selection, design and implementation.

Does GHL require sophisticated IS?

Small firms do the same (broadly similar) type of work as larger firms, and therefore need much the same type of software to assist them in planning. For example, Oliver Wight's universal manufacturing equation tells us the same for material requirements planning. Irrespective of the firm's size, this equation holds true. One has to answer four simple looking questions (which firms have found practically difficult to answer). These questions are -

1. What are we going to make?

2. What does it take to make it?

3. What do we have?

4. How much to procure / manufacture?

Though one may agree to the above equation, just as each business is different, the degree to which automation/computerisation is required is different. Certainly the needs of GHL are not same as those of a larger firm like Indian Scooters Ltd. which relies heavily on the voluminous

Data and requires extensive inventory tracking.

The investment needs of sophisticated planning software may not be justifiable at this juncture. GHL could continue with the present paperwork based or manual systems.

In the coming months/year GHL is expecting rapid growth in terms of more products and volumes. This will increase the complexity of operations because of common resource requirement for products. This is when GHL will require a formal planning system or even a computerised system. One key point here would be - to what extent to automate the business. Before any investment is made, it's crucial to identify the firm's specific needs and tailor the solution accordingly. Following should be clearly identified - which data is relevant, what reports one should be able to get from the system, what kind of records one should maintain, what data to capture

Areas where IT solutions can be deployed by GHL:

As in the case of many other companies, it would be easier to computerise support functions such as billing/finance/accounting first. Some off-the-shelf standard packages available in the market can be used, e.g. Tally, EX-next generation. These softwares are very cheap and easy to use.

Purchasing/Stores/Materials handling should be the next activity to be computerised.

MRP kind of system may be helpful for Shirish's operations, given the similarity in the structure of the products. Such system will help at least in coordinating the release of purchase orders and production orders, if not more. Typical steps involved would be -

1. Item numbering / coding

2. Inventory classification of items

3. Bill of Material (BoM)

4. Creating a vendor database file (this can also be used for tracking their performance on price, qulaity, deliveries)

5. Estimation of procurement and manufacturing lead times

6. Ordering and stocking norms

7. Record keeping of regular transactions

8. Record keeping for monitoring the inventory status of each item

One of simplest and easiest thing for successful implementation of this system is to have 100 % accuracy of BoM and inventory data files. For correct inventory status the stores in-charge should record everything that goes in and out of the stockroom. Periodic cycle-counting should be done to rectify any discrepancies.

Another important thing is - apart from the financial costs (cost of purchase, installation and support for software / hardware), there is a serious need for I.T. skills and experience to be available in-house. It is simply impossible to purchase a system (however expensive) and get it up and running without investing a major input of time and skill of the people in the firm. Some training activity is a must before further steps towards computerisation.

Medium-to-long term planning

Capacity planning

Although some improvements in the existing system are possible through improvement in work practices, capacity expansion should be planned for the rapidly growing demand over next few months/year.

Firstly, there is only one cutting & stripping machine at GHL.  A back-up machine for this operation is urgently required to take care of scenarios of unplanned break-down and repairs.

Since first two operations - cutting/stripping and crimping - are performed by automatic/semi-automatic machines with low labour content, their capacities and quality of output are not sensitive to changes in work practices.  Therefore their capacities should always be kept balanced and slightly under-utilised so that they can absorb short-term fluctuations in the demand.  This is important also because the capacity of these machines can be increased only in certain chunks and would require significant investments.  This strategy, though requires good financial management on part of GHL, would be wise in the long run since the increasing trend in demand is almost certain to continue for at least a year.  It will give GHL an edge (over competitors) to respond quickly to new orders or design changes.  Hence, GHL could seriously consider purchasing one cutting/stripping machine and one crimping machine of similar capacities as the existing ones as early as possible.

There is no need to hire excess manpower right now as labour is easily available from nearby areas.  There should be greater emphasis on training the existing labour in as many work areas as possible, and retaining them for long-term by designing a compensation package and incentive scheme that would provide their families enhanced economic security.

Since the product variety is soon going to increase, some thought should be given to a systematic storage of items.  Designing a formal material storage and control system would help GHL to avoid confusion due to increase in variety and volumes and simultaneously reduce costs.  So better management of the existing storage space and planning for additional storage space for future needs can be started.

Technology

Since the sub-assembly and final assembly operations are likely to remain largely manual even in future, going for large capacity automatic machines for cutting, stripping and crimping may not increase the productivity significantly.  The sub-assembly operation, because of its very nature, is likely to remain the constraining operation (or bottleneck) even in future.  So the effort should be to enhance the effectiveness of this operation by breaking it into a sequence of more than one work-elements, and by improving the design of fixture for the same.  The capacity of all other operations (including cutting, stripping and crimping) should be more than the capacity of sub-assembly but not too much.  The effort should be on developing many well-trained workers who can potentially work in the sub-assembly area.  Even though some of these workers may be engaged in some other work area (like cutting/crimping, final assembly, inspection or stores), sudden increases in demand could easily be absorbed (without compromising quality) by shifting the workforce to the constraining operation.  Hence, ‘flexibility of workforce’ should be the key strategy for attaining technological superiority.

Finance

Since the capacity of cutting, stripping and crimping operations are to be always kept slightly underutilized, this would require wise management of finances, in particular raising the working capital as and when required.  Building good relationship with financing agencies is therefore important.  Also, continuous effort should made to keep inventories to the minimum, in order to avoid locking of precious working capital.  Working capital should be used only for productive and value-adding investments (like adding capacity) and not for holding inventories.  A formal system (preferably computerized) for inventory control would go a long way towards this end.