### INTRODUCTION TO AIR BRAKE

Air brake stock was first introduced on Indian Railways in 1968 on the Waltair-Kirandul (KK) line of SE Railway with 300 sets of imported air brake equipment fitted in BOI wagons. In 1978 Railway Board constituted a high level committee to study the various aspects of application of air brake on Indian Railways. Induction of air brake trains on trunk routes of Indian Railways started in 1982 with the introduction of BOXN wagons.

Air brake system has certain advantages over vacuum brake system. In air brake compressed air is used to produce braking force. Distributor valve is the key equipment to control air brake system. Performance of air brake largely depends on distributor valve. It has certain advantages over vacuum brake system. In air brake compressed air is used to produce braking force. Distributor valve is the key equipment to control air brake system. Performance of air brake largely depends on distributor valve.

Advantage of air brake:

- The emergency braking distance with the trailing load of 4500 ton and 75-km/h speed is 870 mt. as compared to 1780 mt. In vacuum brake.

- There is no brake power fading. Automatic re-feeding of brake cylinder pressure loss is possible.

- Repetitive brake application is possible. It is possible to maintain the speed from 75 km/h to 10 km/h while negotiating 1 in 100 down gradient without stopping the train.

- Brake propagation at 286 m/sec is possible.

- Automatic re-feeding of brake cylinder pressure loss is possible.

- Formation of longer train is possible.