SUGGESTIVE MAINTENANCE PRACTICES TO REDUCE INCIDENCE OF BRAKE BINDING IN AIR BRAKED COACHING STOCK

M.K.De, lecturer/IRIMEE

# Defining brake binding:

- This is defined as the situation when brake block is in contact with wheel tread while Driver’s brake valve (A- 9) kept in RELEASED position.

# Avoiding confusion! ! !

- Very often incidence of wheel skidding is referred as brake binding. Wheel skidding and brake binding are two different phenomenons.

- Force required to roll a brake binded wheel is less than to roll it.

- There will be increase in tractive effort to roll a binded wheel that depends on the point of pulling the wheel, acceleration and the adhesion. There is 20 – 40% increases in tractive effort for all practical condition.

Prevention is better than cure:

Instead of analysing a specific case of brake binding, it is worth to analyse the factors and the related maintenance practices that influence the incidence of brake binding. From the fundamental laws of statistics, 90-95 % of the actual cases of brake binding is due to the COMMON CAUSES and only 5 –10 % are due to SPECIAL CAUSE.

The steps involved are:

- Identify the FACTORS related to the incidence of brake binding.

- Identify the PRACTICES influencing these FACTORS

- Identify the SPECIAL ASPECTS in the maintenance practices.

Following table suggests the Factors and related maintenance/handling practices that influence the incidence of brake binding in most of the cases. The special aspects to be specially checked during maintenance have also been highlighted.

| Factors | Related maintenance practice | ## Aspects to be specially checked |
| --- | --- | --- |
| Defects in Locomotive | Inspection of air system | ## Check for fluctuation in BP pressure. - Check for moisture/oil / dirt content in compressed air. |
| Defects in train handling | Releasing practice of brake | ## Wait for one minute at least after moving brake valve handle to release position after emergency application. - Avoid extended application of the pressure surge to avoid over charging |
|  | Observing air flow meter | ## Keep close watch in the airflow indicator to monitor incidence of excess leakage. |
|  | Observing ampere meter | ## Keep close watch in the Ampere meter reading for high current |
|  | Controlling miscreant activity | ## Keep vigilant on tampering DV isolating handle position |
|  | Drainage of condensate | ## Drain condensate from air reservoir, oil separator and filter. - Open drain cock slowly so that condensate is carried out with the air. |
| Defects related to DV | Mounting of DV on common pipe bracket | - Check holes of common pipe bracket are one-to- matched with the corresponding location on the CPB seal. - Check collars provided on the CPB seals are uniformly projected. - Use small quantity of hard grease on CPB before positioning of the seal to avoid displacement while mounting. - Check that release choke is clear. |
|  | Assembly of DV with Intermediate flange | - Overhaul / Repair distributor valve with Intermediate flange as a single unit. - Check holes of Intermediate flange seal are one –to – one matched with respective location in the distributor valve. - Check collars provided on the seal are uniformly projected. |
|  | Inspection of DV | - Check the valve is free from rubber buldging or debonded for valve plate (10) in KE type or valve (37) in C3W type. - Check for smooth movement of hollow stem in its guide during assembly. This may be ascertained by: - Movement of hollow stem Check that hollow stem is not bent. - Check that outer surface of the hollow stem is polished. - Apply light coat of thin grease on the hollow stem. - Check the condition of CPB filter. This is ascertained by: - Clean CPB filter in every schedule. - Clean vertical passage in CPB. |
| Defect in Brake cylinder | Inspection of brake cylinder | - Check for breathing passage. This is ascertained by: - Avoid greasing of Trunk during assembly. - Clean annular space between Trunk and Front cover. |
| Defects in Dirt collector | Inspection of dirt collector. | - Condition of outlet passage. This is ascertained by: - Clean Dirt collector in every schedule. |
| Defects in SAB brake regulator | Inspection of Barrel | - Check that the barrel has no dent. - Confirm that Traction sleeve passes smoothly inside the entire length barrel during assembly. |
|  | Inspection of Adjuster nut | - Check axial play of the Adjuster nut with the spindle. Since actual measurement is difficult, excessive play must be felt while assembly. - Make sure that the adjuster nut falls freely along the spindle when held vertical. |
| Defects in Brake gear | Adjustment of brake gear | - Adjust end pull rod hole and maintain length of pull rod such that the Equalising levers be in near vertical in brake applied position. |
|  | Inspection of Equalising lever | - Ensure that the Z-lever pins used with Equalising levers are polished. - Ensure the corresponding bushes are without any oblong deformation. - Radial clearance is maintained within 0.75 mm. |
|  | Inspection of Actuating rod | - Ensure that the Actuating rod is not twisted. The straightness is realised by smooth fitment of the Actuating rod with the both Equalising levers with the pins. |