# INVESTIGATION PROCEDURE FOR BRAKE BINDING WITHOUT FORMAL TESTING

By: M.K.De, Lecturer/IRIMEE

## Definition:

Brake binding is defined as the situation when the brake block is in contact with the wheel tread though the A-9 valve position is in released position. The severity of the brake binding depends on with how much force the brake block is in contact with the wheel tread.

- Confusion!

Very often wheel skidding is wrongly interpreted as brake binding. Though brake application quality in relation with the adhesion is the basic cause of wheel skidding, the incidence of wheel skidding must not be merged with the incidence of brake binding.

- Preparation:

- Be careful that no evidence is lost in the process of investigation such as pulling QRV or isolating DV.

- Record relevant information like Coach No., DV make, whether isolated, whether SAB regulator fitted, schedule particulars etc. These may be useful in drawing different conclusions.

- It is possible to identify the cause of brake binding and to take corrective measures depending on the situation without carrying out formal testing procedure.

NOTE:

This investigation does not include any reason like breakage of train pipe, air hose de-coupling or any other easily noticeable defects.

- INVESTIGATION

The investigation involves questionnaires and its answers with either of two options only. Conclusion is drawn from the option. This is explained in the following table:

| SL | QUESTION | OPTION | CONCLUSION | REMARKS |
| --- | --- | --- | --- | --- |
| 1 | Is it FULL RAKE? | Yes | BP pressure fluctuating in Loco |  |
|  |  | No | Individual coach involved | Follow SL 2 |
| 2 | Is there air in brake cylinder? | Yes | DV defective |  |
|  |  | No |  | Follow SL 3 |
| 3 | Is breathing passage in brake cylinder clogged? | Yes | Brake cylinder defective | Clean breathing passage with kerosene oil and a poker. |
|  |  | No |  | Follow SL 4 |
| 4 | Is A-1 dimension in SAB greater than 98 mm in case of freight stock or 93 mm in coaching stocks? | Yes |  | Follow SL 5 |
|  |  | No |  | Follow SL 6 |
| 5 | Is A- dimension zero? | Yes | SAB may or may not be defective |  |
|  |  | No | SAB defective | Also follow SL 6 |
| 6 | No question! Check brake gear components thoroughly |  |  |  |

- CHECKING OF BRAKE GEAR COMPONENTS

Identifying defective brake gear components needs very careful inspection. This is because of the fact that these components are not equipment and has no malfunctioned symptom. Following are some important areas to be looked into.

- Check that the actuating rod is not twisted or grazing with the bolster.

- Check that all the brake gear pins are not scored or rusted or jam in their respective holes.

- Check that the SAB pull rod is not entangled with its bracket or bogie headstock.

- Check that the equalising levers (at least two in each bogie) is in near vertical position in application position.

- Check that the hand brake wheel is not on.

- Check that the pull rod is dimensionally correct.

- Additional for freight stock:

- Check that the sleeve nut is properly set.

- Check that the pocket type brake beam is jammed in its pocket

NOTE:

These are not exhaustive checkpoints. The Inspecting official must keep watch on other relevant factors on particular incident.