TROUBLE SHOOTING FOR C3W TYPE DISTRIBUTOR VALVE

Followings are most frequent failures in C3W type Distributor valve. Effects of such failures with significant physical identification to locate these failures and their probable causes are tabulated for ready references.

| SL | DEFECTS | EFFECTS | CAUSE OF FAILURE |
| --- | --- | --- | --- |
| 1 | O-ring (32) loose in the groove of Lower cover (2a) | CR & AR vent to atmosphere from the joints Lower cover & Body | Poor machining of lower cover, dimensional inaccuracy of O-ring |
| 2 | Diaphragm (6) displaced from clamp (10) | CR & BP vent to atm. through ‘S’ hole | Wrong fitment of diaphragm, poor machining of diaphragm clamp, wrong fitment of lip seal, permanent set of the diaphragm |
| 3 | Diaphragm (27) displaced from clamp (28) | BC vents to atm. through ‘S’ hole | Wrong fitment of diaphragm, poor machining of diaphragm clamp, wrong fitment of lip seal, permanent set of the diaphragm |
| 4 | Pin (2d) breaks inside Lower cover | Manual pull with single pull is not possible | Excessive reaction force, proposal to RDSO made by the author in 1995 and approved, manufacturers yet to comply |
| 5 | Falling of double release valve components/ slippage of circlip (17) from its groove in lower cover | Manual release not possible | Bad handling, poor machining of groove for the circlip |
| 6 | Jamming of solex jet (66) | CR charging not possible | Contaminated air |
| 7 | Lip seal (218) of the Quick service valve deformed or cut | CR vents to atm. through ‘F’ hole | Poor machining or excessive clearance between end wall and lip seal, permanent set in lip seal |
| 8 | Lip seal (24) of diaphragm holder (3) deformed or cut | BP vents to atm. through ‘S’ hole | Poor machining or excessive clearance between end wall and lip seal, permanent set in lip seal |
| 9 | Lip seal (24) of lower cover (2a) deformed or cut | CR vents to atm. through release choke | Poor machining or excessive clearance between end wall and lip seal, permanent set in lip seal |
| 10 | Valve (11) of double release valve cut or excessive grooved | CR vents to atm. through double release choke | Soft rubber of the bonded valve |
| 11 | Pusher pin (79) short in length | CR charging not possible | Poor machining |
| 12 | O-ring (81) on the guide of cut off valve assly cut | CR flows to BC line causing brake binding | Soft rubber quality |
| 13 | Hex. Head of diaphragm nut assly (82) of cut off valve assly with inadequate height | Difficulties in assembly /disassembly of cut off valve assly | Improper machining |
| 14 | Inside of DV & other components rusty / emulsification of grease | Any form of malfunction may occur | Contaminated air |