DIFFERENCE BETWEEN C3W AND KE TYPE DISTRIBUTOR VALVE

# By: M.K.De, Lecturer/IRIMEE

# Differences in description of major sub assemblies:

| SL | EK sub assembly | Equivalent C3W sub assembly |
| --- | --- | --- |
| 1 | R-charger assembly | Isolating handle assembly Check valve assembly Double release valve assembly(partly) Cut off valve assembly(partly) |
| 2 | Minimum pressure limitter | Inshot valve assembly |
| 3 | Maximum pressure limitter | Limiting device |
| 4 | A-controller | Cut off valve assembly(partly) |
| 5 | U-controller | Quick service valve assembly |
| 6 | Top cover valve | Main valve assembly |
| 7 | Quick release valve | Double release valve assembly(partly) |

- Differences in major working processes:

| SL | Process description | EK distributor valve | C3W distributor valve |
| --- | --- | --- | --- |
| 1 | Isolation of distributor valve | - Entry of brake pipe pressure from CPB side is stopped. - AR pressure from distributor valve vents to atmosphere | - Entry of brake pipe pressure from CPB side is stopped. - BP pressure from distributor valve vents to atmosphere |
| 2 | Quick service application | BP pressure from DV vents to atmosphere during the initial stage of brake application | BP pressure from DV vents to atmosphere during the final stage of brake release |
| 3 | Manual release | Only CR pressure releases directly while pulling QRV manually(AR pressure vents to atmosphere while isolating DV) | It is possible to vent CR and AR pressure while pulling Double release valve manually |