# METHOD OF DISTRIBUTOR VALVE ISOLATION

## By: M.K.De, Lecturer/IRIMEE

Following procedure is to be followed while isolating a distributor valve when necessary.

- Close AR isolating cock. This will not be applicable in single pipe working system.

- Isolate distributor valve. This is done by keeping isolating handle of the distributor valve in horizontal position. It is preferred to tie the handle tightly with wire in isolated position.

- Pull Quick release valve. Ensure that BC pressure is released. This is essential as in most practical cases  BC pressure is not released with single pull of the QRV, though it is supposed to be so.