#### MAINTENANCE PROCEDURE OF DISTRIBUTOR VALVE

#### By: M.K.De, Lecturer/IRIMEE

- The procedures for maintenance of distributor valve are to be followed as follows:

- Recording incoming particulars such as DV serial no., make, and incoming station.

- Record deficient, broken components visible externally.

- Ensure DV with Intermediate flange as a single unit.

- External cleaning

- Remove dust with soft wire brush.

- Clean DV with compressed air/water jet.

- Remove water accumulation by dry compressed air.

- Internal cleaning

- Dismantle complete assembly. Keep different sub assemblies in separate container to prevent mixing of parts.

- Clean metallic components in kerosene oil. Wipe out excess oil by clean cotton cloth. Clean rust residue on the housing with soft wire brush or kerosene dipped cotton cloth. Avoid excessive rubbing to protect wax coating.

- Clean rubber bonded and rubber components by clean cloth soaked in spirit. In case of non-availability of spirit, cleaning by cloth soaked in plain water may be done. Kerosene must not be used for cleaning of rubber components.

- Assembly

- Clean all passages /chokes /breather holes by dry compressed air jet before assembly.

- Replace mandatory replacement items(POH kit).

- Replace other visually defective items.

- Fit deficient items.

- Record particulars.

### CAUTION

All sliding parts guide members, seals. notched segments must be lightly greased and to check for smooth movement.

Grease lightly the position of the diaphragm that goes into the groove or housing for proper assembly.

Rubber bonded parts must be free from swelling and deep groove mark.

- Testing

- Set brake pipe pressure at 5 kg/cm2. Perform entire testing in single pipe working. Rectify the defects observed during testing. Record all the test parameters. Perform the following test.

- Isolating test: Mount DV on test bench. Keep DV in isolated position. Open compressed air line and keep Driver’s brake valve in charging position. Wait for five minutes. There must not be any rise of CR or AR pressure.

- Air tightness test: Keep isolating handle in normal position. Charge the system for at least five minutes. Apply soap solution along the joints with soft nylon brush. There must not be any air bubble.

- CR /AR charging time: Empty distributor valve. Recharge the system. During the process of charging, check time required to rise of CR/AR pressure from 0 to 4.8kg/cm2. This must be within 160 to 280 seconds.

- Brake application time: Drop brake pipe pressure by 1.6 kg/cm2. Check time required to rise of BC pressure from 0 to 3.6 kg/cm2. This should be within 3 to 5 seconds for coaching stock and be within 18 to 30 seconds for freight stock.

- Maximum BC pressure: In continuation to above, check the max. BC pressure attained. It should be within 3.8  0.1 kg/cm2. Further drop of BC pressure will not affect in increase of BC pressure.

- Brake release: Recharge BP pressure to 5 kg/cm2. Check time to release of BC pressure from 3.8 to 0.4 kg/cm2. This should be within 15 to 20 seconds in coaching stock and 45 to 60 seconds in freight stock.

- Uniform brake propagation: Recharge the system. Apply and release the BC pressure. Feel venting of BP pressure to atmosphere as below:

- For EK type: During the initial stage of brake application, BP pressure vents to atmosphere through port no. 2 of the distributor valve. The venting is stopped when BC pressure reaches to about 0.8 kg/ cm2.

- For C3W type: During the last stage of BC release, BP pressure vents to atmosphere “S” hole.

- Gradual brake application: Recharge the system. Drop BP pressure gradually from 0 to 1.6 kg/cm2. BC pressure must rise in proportion to drop of BP pressure. Perform the test for at in six steps.

Thumb rule: For every drop of 1 kg/cm2 of BP pressure, there will be rise of 2.2 to 2.5 kg/cm2 of BC pressure subject to maximum drop of 1.6 kg/cm2 of BP pressure w.r.t 5kg/cm2.

- Sensitivity test: Recharge the system. Close main supply. Open Sensitivity choke. (This is in-built choke correspond to drop of BP pressure @ 0.6Kg/cm2/6 sec.) There must be immediate rise of Brake cylinder pressure.

- Insensitivity test: Recharge the system. Close main supply. Open Insensitivity choke. (This is in-built choke correspondent to drop of BP pressure @ 0,3 kg/cm2/min.) CR pressure must drop along with BP pressure and there must not be any rise of BC pressure. Wait for at least 30 seconds.

- Backflow test: Recharge the system. Close main supply. Apply emergency brake. Wait for one minute. Check that there is no drop of CR, AR and BC pressure or increase of BP pressure. Be careful to observe the pressure changes.

- Manual release test: In continuation to above test(with emergency application), make a single pull to quick release valve. Check that CR and BC pressure drops to zero.