# TROUBLE SHOOTING FOR KE TYPE DISTRIBUTOR VALVE

Followings are most frequent failures in KE type Distributor valve. Effects of such failures with significant physical identification to locate these failures and their probable causes are tabulated for ready references.

| SL | DEFECTS |  | EFFECTS | CAUSE OF FAILURE |
| --- | --- | --- | --- | --- |
| 1 | O-ring (12) of top cover cut |  | AR flows to BC and ATM through release choke simultaneously in released condition | ##### Wrong assembly |
| 2 | Grooved ring (25) in lower part of guide piece cut |  | BP flows to ATM through release choke | Excessive clearance, poor machining of its housing, permanent set of rubber |
| 3 | Grooved ring (49) of R-charger assly cut or perished |  | BP flows to ATM through R-charger vent hole | Excessive clearance, poor machining of its housing, permanent set of rubber |
| 4 | Grooved ring (54) of main body assly cut or perished |  | BP flows to ATM through port no. 3 | Excessive clearance, poor machining of its housing, permanent set of rubber |
| 5 | Grooved ring (49) of A-controller assly cut or perished |  | BP flows to ATM through port no. 2 | Excessive clearance, poor machining of its housing, permanent set of rubber |
| 6 | O-ring (85) in bottom cover cut or perished |  | CR flows to ATM through bottom cover and main body joints | Wrong assly, permanent set of rubber, poor machining of groove in bottom cover |
| 7 | Grooved ring (35) of A-controller assly cut or perished |  | CR flows to ATM through port no. 4 | Excessive clearance, poor machining of its housing, permanent set of rubber |
| 8 | Grooved ring (35) of maximum limitter assly cut or perished |  | AR flows to ATM through port no. 4 | Excessive clearance, poor machining of its housing, permanent set of rubber |
| 9 | Grooved ring (35) of minimum pressure limitter assly cut or perished |  | AR flows to ATM through port no. 4 | Excessive clearance, poor machining of its housing, permanent set of rubber |
| 10 | Grooved ring (35) of U-controller assly cut or perished |  | BP flows to ATM through port no. 1 in application position | Excessive clearance, poor machining of its housing, permanent set of rubber |
| 11 | K-ring (101) in QRV assly cut or perished |  | CR flows to ATM through vent passage of QRV | Bad handling in case of deficient QRV, permanent set of rubber |
| 12 | Valve plate (10) in top cover assly grooved or buldge |  | AR flows to BC in charging condition or BC can not flow to ATM in released condition | Poor bonding, poor quality of rubber |
| 11 | Diaphragm (16) of main body assly cut |  | BC flows to ATM through port no. 4 | Poor rubber quality |
| 12 | Diaphragm (99) of bottom cover assly cut |  | CR and BP isolation not possible resulting ‘No application’ | Poor rubber quality, excessive stress on rubber by support plate (100) |
| 13 | Diaphragm (69) of side cover assly cut |  | BC vents to ATM through port no. 4 | Poor rubber quality, rough surface of side cover(165) |
| 14 | Sealing cap (133) of R-charger assly cut |  | AR and BP isolation not possible resulting failing in ‘Sensitivity test’ | Poor rubber quality, rubbing effect of spring plate (134) |
| 15 | Choke (117) of bottom cover assly jam |  | CR charging not possible | Contaminated air |
| 16 | Choke (17) of top cover assly jam |  | BP flows to ATM through port no. 2 during application | Contaminated air |
| 17 | Choke (175) jam |  | Delay in brake application | Contaminated air |
| 18 | Choke (172) jam |  | Delay in brake application or brake binding | Contaminated air |
| 19 | Seal (196) perished |  | Any form of malfunction may occur | Wrong assembly |
| 20 | Seal (77) perished |  | Any form of malfunction may occur | Wrong assembly |
| 21 | Inside of DV & other components rusty / emulsification of grease | Any form of malfunction may occur |  | Contaminated air |