# BRAKE SPECIFICATION AS PER RDSO PAMPHLET 02 ABR-94

| SL | Parameters | Standard value |
| --- | --- | --- |
|  | Leakage in BP line | 0.1 kg/cm2 |
|  | Leakage in FP line | 0.1 kg/cm2 |
|  | Normal running | BP=CR= 5±0.1, AR=5±0.1 for single pipe and 6±0.1for twin pipe, BC= 0 |
|  | CR charging | 260±20 sec |
|  | AR charging | 270±30 |
|  | BC rising from 0 to 3.6 kg/cm2 | 3 to 5 sec for coaching stock 18 to 30 sec for freight stock |
|  | Maximum BC pressure | 3.8± 0.1 kg/cm2 |
|  | Corresponding BP pressure for maximum BC pressure | 3.7 to 3.4 kg/cm2 |
|  | BC pressure draining time from 3.8 to 0.4 time kg/cm2 | 15 to 20 sec for coaching stock 45 to 60 sec for freight stock |
|  | Sensitivity test | Brake should start applying within 1 sec for coaching stock And 6 sec for freight stock |
|  | Insensitivity test | Brake should not start applying |
|  | Working of manual release valve | CR excess of BP should vent and BC pressure releases |
|  | Effect on chain pulling |  |
|  | Effect on guard’s emergency application |  |