## MAINTENANCE PRACTICE OF SAB BRAKE REGULATOR

### By: M.K.De, Lecturer/IRIMEE

Following procedure may be followed during the maintenance of SAB type brake regulator.

- Dismantle defective brake regulator

### CAUTION

- Clamp brake regulator barrel lightly in a pipe vices near barrel head to avoid damage to the barrel

- Wash all the parts in kerosene by nylon brush.

- Inspect all the components

- Check some of the vital components as below:

- Ensure that the ball cage must not rub against either of the ball race and leader nut flange under normal hand pressure.

- Ensure that the corners of friction washer be sharp.

- Ensure that the mating surface of actuating sleeve with that of the socket and adjuster nut be free from any burr and scoring.

- Ensure that the slot provided in the actuating sleeve be free from formation of any groove.

- Ensure that the barrel with head is not loose and the barrel is not flattened at any place. Check that the traction sleeve travels freely in the entire length of the barrel.

- Ensure that the axial play of the adjuster nut and the leader nut with the spindle be within 2 mm.

- Check that the adjuster nut and the leader nut fall freely by their own weight along the thread of the spindle when held vertically.

- Check that the free height of the barrel spring be within 600-620 mm for freight stock and be within 410-425 mm for coaching stock.

- Check that the spring sleeve is not elongated.

- Replace defective components.

- Apply semifluid servogem-2 or equivalent grease in all the components.

- Assemble the brake regulator except spindle and safety collar.

CAUTION:

- Place adjuster nut in the traction sleeve carefully to avoid tilting.

- Clutch ring should be in contact with the clutch sleeve. It must not be in contact with the ball bearing.

- Feel a clearance of 2-3 mm approx. by pressing the adjuster tube after assembly of the traction unit.

- Perform testing with the test rig.

- Maintain air pressure at 4 kg/cm2.

- Perform take up test:

Ensure that the brake regulator takes up maximum 100 mm per brake application.

- Check that the A-1 dimension be maintained at 98 mm for freight stock and 93 mm for coaching stock.

- Perform pay out (pay in ) test :

Ensure that the brake regulator pays out maximum 30 mm per brake application.

- Repeat take up and pay out test at least for three times.

- Remove test spindle and fit pre-inspected and greased spindle and then fit safety collar

on the spindle.

- Record test observations and repair particulars in the prescribed format.

- Place repaired and tested O.K brake regulator in the storage rack.                          .

.