PRINCIPLE OF OPERATION OF SAB BRAKE REGULATOR

By M.K.DE, LECTURER/IRIMEE

# Introduction

A brake system can be grouped in

- Pneumatic system and

- Brake gear system.

Performance of pneumatic system largely depends on Distributor valve and that of brake gear system on SAB brake regulator. The working principle of SAB brake regulator lies on the NON-SELF LOCKING property of thread profile of the spindle, leader nut and adjuster nut.

Purpose of SAB brake regulator:

The function of SAB brake regulator is to maintain constant clearance between brake block and wheel tread.

### Terminology:

### There are few terms associated with the working SAB.

- A-dimension: This is the clearance between control rod head and barrel-head face in brake released condition. This can be termed as INDEXED clearance between brake block and wheel tread. Its value depends on.

- Bogie leverage ratio.

b) Number of Bogies.

c) Number of SAB brake regulator.

d) Brake block clearances.

This A-dimension is manually set. The values for different stocks:

Coaching stock =20 mm.

CASNUB fitted wagon =70 mm.

UIC fitted wagon =50 mm.

The working formula to evaluate A-dimension:

A=    Bogie leverage X No. of Bogie X Brake block clearance

#### No. of SAB

| #### STOCKS | #### Bogie #### leverage | #### No .of bogie | #### No. of SAB | #### Brake Block #### clearance | #### A-dim. |
| --- | --- | --- | --- | --- | --- |
| #### ICF Coach | #### 4 | #### 2 | #### 2 | #### 5.0mm | #### 20mm |
| #### CASNUB Bogie | #### 6 | #### 2 | #### 1 | #### 5.9mm | #### 70mm |
| #### UIC Bogie | #### 4 | #### 2 | #### 1 | #### 6.25mm | #### 50mm |

Take up operation: This is the mechanism by which SAB brake regulator brings down the excess slack of the brake gear to normal slack.

Pay out (pay in) operation: This is the mechanism by which SAB brake regulator brings up the less slack of the brake gear to normal slack.

e-dimension: This the maximum amount in millimetres that a SAB brake regulator can shorten its overall length during TAKE-UP operation, when wheels .brake blocks, and other brake gear items are new.

### Working principle (Take-up operation)

- Initial stage application

In the first stage of application whole assembly moves towards control  rod head by an amount equal to it’s A-dim. In case of excess slack, the barrel will be held against the control rod head, but brake block will not able to touch the wheel tread.

Intermediate stage application

As the brake block is not touching the wheel, free expansion of the brake cylinder will try to pull out adjuster tube. The force transmission will be as below.

↓ Adjuster tube

↓ Actuating sleeve

↓ Clutch spring

↓ Clutch sleeve

↓ Traction sleeve

↓ Adjuster nut –Adjuster nut locked by traction sleeve & is restricted to rotate.

↓ Spindle

↓ Leader nut –Leader nut rotates on bearing, but restricted to move forward.

As the leader rotates at the same place, the spindle is driven in. The spindle will be driven in by the amount of excess slack present in the brake rigging. This is TAKE-UP operation. The outside end of the spindle is welded with pull rod. This results in pulling of pull rod and in turn all brake gear components comes in action, The spindle and the pull rod is to move continuously till brake block is held against the wheel tread. Movement of brake rigging is stopped. The effective length of the spindle will be shortening corresponding to excess slack present in the brake rigging.

- Final stage application:

Brake cylinder pressure starts building up. The clutch spring is compressed. The adjuster tube moves corresponding to compression of clutch spring. This opens contact between clutch ring and clutch sleeve. The traction unit rotates and advances on the spindle. The brake cylinder piston also advances without affecting any brake rigging movement. The piston stroke will be restricted when combined barrel spring and clutch spring force will balance the brake cylinder force.

- Initial stage of release:

When the brake cylinder pressure starts diminishing, so far brake cylinder pressure is sufficient to keep gap in clutch ring and clutch sleeve, whole barrel including traction unit rotates on the spindle against thrust bearing of clutch sleeve. The adjuster tube moves inside the barrel without affecting brake-rigging movement.

Intermediate stage of release:

When the brake cylinder pressure further diminishes that it is not able to keep clutch open, the barrel stops rotating on the spindle. The barrel spring still pushes the traction sleeve and the adjuster nut becomes free. The take up spring pushes the adjuster nut. The adjuster nut rotates on the spindle and simultaneously moves towards the leader nut along with traction unit. It continues to do it till the adjuster nut is held against spring sleeve of leader nut casing assembly. In this stage, the adjuster tube moves inside the barrel affecting the brake rigging movement to partial release. Internal functions are completed in this stage.

- Final stage of release:

The compression spring of the brake cylinder tries to move complete brake rigging. The whole SAB brake regulator moves to release brake rigging completely. In this stage A-dimension is regained.

IMPORTANT NOTE

- Take up operation is performed during brake application

- Take up is retained during brake release

- In normal braking, barrel does not rotate during brake application. The barrel rotates during early stage of brake releasing. In this stage, physical movement of brake rigging does not take place.