### AIR BRAKE SYSTEM OPERATION

LAY OUT:

DESCRIPTION:

The compressor in the locomotive produces the air supplied to the system. It is stored in the main reservoir. Regulated pressure of 6 kg/cm2 flows to the feed pipe through feed valve and 5-kg/cm2 pressure by driver’s brake valve to the brake pipe. The feed pipe through check valve charges air reservoir via isolating cock and also by brake pipe through distributor valve.

The brake pipe pressure controls the distributor valves of all the coaches/wagons which in turn control the flow of compressed air from Air reservoir to brake cylinder in application and from brake cylinder to atmosphere in release.

During application, the driver in the loco lowers the BP pressure. This brake pipe pressure reduction causes opening of brake cylinder inlet passage and simultaneously closing of brake cylinder outlet passage of the distributor valve. In this situation, auxiliary reservoir supplies air to brake cylinder. The application time, pressure in the brake cylinder and other brake characteristics are controlled by distributor valve.

During release, the BP pressure is raised to 5 kg/cm2 . This brake pipe pressure causes closing of brake cylinder inlet passage and simultaneously opening of brake cylinder outlet passage of the distributor valve. The distributor valve connects brake cylinder to atmosphere.

The brake cylinder pressure can be raised or lowered in steps.

In case of application by alarm chain pulling, the passenger emergency alarm signal device (PEASD) is operated which in turn actuates passenger valve (PEV) causing exhaust of BP pressure through a choke of 4 mm.

Opening of guard emergency brake valve also makes emergency brake application.