BRAKE CHARACTERSTICS OF AIR BRAKE PASSENGER TRAIN IN ACCORDANCE WITH UIC LEAFLET 540 ANNEXURE 3f

- CR filling time in a single coach from 0 to 4.8 bar be within 160-210 seconds.

- AR filling time in a single coach from 0 to 4.8 bar be within 140-240 seconds.

- Filling time of brake cylinder up to 0.95 of maximum pressure with or without slack adjuster be within 3-5 seconds.

- Release time of brake cylinder down to 0.4 bar with or without slack adjuster be within 16-20 seconds.

- Maximum brake cylinder pressure be within 3.6 to 3.8 bar.

- Full application is reached on reduction of BP pressure by 1.5 bar.

- Automatic re-feeding of brake cylinder pressure loss is to be provided.

- Brake in single coach must not respond on brake pipe pressure reduction @ 0.47 bar in 60 seconds.

- Brake in train of 80 axles must not respond on brake pipe pressure reduction @ 1 bar in 384 seconds.

- Brake in single coach must respond on brake pipe pressure reduction @ 0.6 bar in 6 seconds.

- Length of pressure surge of 6 bar after full application without over charging be 14.5 seconds.

- Release of train of 60 axles all braked tare condition after full application be within 20 seconds.

- Speed of transmission in a train of 80 axles tare condition be 286 m/sec.

- There must not be any reduction of brake power.