SEMINAR ON

RELIABILITY OF C&W STOCKS, CORROSION REPAIR & PREVENTION AT KGP FROM 3rd Dec 2001 to 5th Dec 2001.

Topics: Scope of improvement in reliability of coaching bogies

by improving in maintenance system.

By: P.R.Prasad, CI / IRIMEE.

1.	INTRODUCTION:

Most of you are well aware about the aim of improving reliability and factors effect the reliability. We will be discussing about the scope of improvement in reliability of coaching bogies by improving in maintenance system.

No doubt, coach is a system of a number of sub-systems, which in turn are built by from numbers of components. So reliability of coaching stocks depends upon the reliability of its individual components and critical sub-assemblies of coaches and method of assembly of these components. For this, it becomes very tough to identify the basic causes towards the incidences of such failure because it may be influenced by the other facts.

- OBSERVATIONS:

I have collected the data of en-route coach detachment of INDIAN RAILWAYS, which are as follows-

Cause-wise analysis of En-route coach detachments of INDIAN RAILWAYS

| ## SL.No. | ## Defects | ## % failure in year |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ## 97-98 (From Mar 97) | ## 98-99 | ## 99-2000 | ## 2000-2000 (upto Dec 2000) |
| ## 1. | ## Brake Gear defects. (Air + vacuum) | ## 43% | ## 47% | ## 48% | ## 60% |
| ## 2. | ## Roller bearing failure. | ## 25% | ## 31% | ## 36% | ## 23% |
| ## 3. | ## Spring breakage. | ## 8% | ## 4% | ## 1% | ## 3% |
| ## 4. | ## Wheel defect. | ## 1% | ## 1% | ## 1% | ## 2% |
| ## 5. | ## Draw & buffing gear defect. | ## 9% | ## 9% | ## 5% | ## 3% |
| ## 6. | ## Bogie suspension. | ## 7% | ## 7% | ## 6% | ## 5% |
| ## 7. | ## Others. | ## 7% | ## 3% | ## 2% | ## 4% |

### As per this failure statistics, it is clearly shown that en-route coach detachments are very high on account of Brake Gear defects & Roller bearing failure.

- REASONS OF HIGH FAILURE OF BRAKE GEAR FITTINGS AND ROLLER BEARINGS:

High failure of Brake Gear fittings & Roller bearings may be due to the any following reasons-

- Design stage.

- Quality of manufacture.

(Manufacturing process, raw material quality & dimensional control).

- Maintainability (Maintenance system).

- Operating factors.

# But out of above these, we are stressing only maintainability and operating factors because the reliability consciousness should be inculcated not only in the officials at the management level but also at the grassroots level. So, I will be giving more stress on human factor as far as reliability is concerned and maintenance procedure at the grassroots level.

# CONCLUSIONS:

- Brake Gear Defects:

Brake Gear fittings are fitted with the underframe of a coach, which are hundred percent inside the track. So for the examination of these fitting, staffs must be entered inside the track and the adjustments / repairs can not be done by one or two staffs. Now we are stressing en-route failures of these fittings.

En-route failure means – these defects could not be detected either by the primary maintenance depots or the secondary maintenance depots. Why it is not detected by these ends? Some questions arise-

- Was it examined by one/two staffs or a group of staffs?

- Was this examinations overlapped by the another group of staffs due to miss-conception?

- What were the type and condition of pit?

- What was the system of pit cleaning?

- What was the lighting arrangement of the pit?

- Was the existing practice to take actual measurement for the purpose of documentation?

- Were the sufficient and good quality tools available?

- Roller bearing failure:

Roller bearings are fitted on axle, which are inside the axle box housing and fully covered by rear & front cover. In normal case, it should not be opened in open line. That is why axle boxes are sealed by POH workshop. But during run axle boxes become loose and water, dust & foreign materials enter into the axle box. Again some questions arise-

- What was the condition of pathways (Specially about slushy or slippery areas) of Pit Line?

- Were the any obstructions to free movement of examining staffs, which examine from out side the track?

- Were sufficient and good qualities of tools available with the individual examining staff?

iii)	General questions for maintenance:

-	What is the existing system to deployment of staffs?

- Is Maintenance Engineer's Diary able to explain proper distribution of examining staffs as far as pin pointing is concerned?

- Is the existing practice to maintain Sch "A" & Sch "B"?

- Is the super checking of maintained rake done honestly, sincerely and regularly by Depot incharge and Divisional levels.

- Whether the examining and inspecting authorities (At any level!) really know in sufficiently & correctly details about items under examination and inspection?

5.	Recommendations:

# What ever I have felt with my job experience that the policies, programmes, modifications, recommended maintenance procedure are not be implemented sincerely and honestly by all concerned due to failure of system inputs. If we are able to design maintenance free C&W assets than we can forget the maintenance system, otherwise we have to strictly provide all these inputs-

# Standardise the deployment of staffs as far as pin pointing is concerned.

- List out the maintenance works, which should be gang/team wise, and not to be over lapped.

- Strictly record actual measurement, not theoretical?

- Strictly provide sufficient and good quality tools.

- Strictly provide clean pit for every time of under gear examination.

- Strictly provide sufficient light for examination?

- Improve in working condition i.e. adequate working space, levelled working area, no obstruction to movement, covering of drains, no slushy or slippery areas etc.

- Every Depot should develop a technical library to clarify any doubts at random and to receive latest information.

At last, ensure quality of repairs because " Higher the quality, higher the reliability."

Thanks to all of you.